Signed Integer Representation

0/1 in the high bit position indicates +/-

Sign-Magnitude Form

High bit stores 0/1 to represent +/-

Low n-1 bits store the magnitude of the integer

Example (1 Byte):

59 : 0011 1011b : x3B

-59 : 1011 1011b : xBB

One's Complement Representation

1Cn(X) = (2n - 1) - X

Equivalent to complementing all bits of X

Example (1 Byte):

59 : 0011 1011  : x3B

-59 : 1100 0100  : xC4

Two's Complement Representation

2Cn(X) = 2n - X

Equivalent to 1Cn(X) + 1, i.e., complement and add 1

Example (1 Byte)

59 : 0011 1011  : x3B

-59 : 1100 0101  : xC5

In practice, Sign-Magnitude representation is never used.

One’s Complement was used in some older machines, but has lost favor to the two’s-complement representation. The difficulty with 1’s complement results form having two representations of 0 (what is  1111 1111b as a signed byte integer?)

2’s Complement Arithmetic

Evaluate 30 – 59 (byte arithmetic)

30	   0001 1110b	x1E

-59	   1100 0101b	xC5

====	   ==========	===

-29	   1110 0011	xE3

Note:

- To form the 2’s complement in hexadecimal, subtract from xFF (byte) or from xFFFF (word). Why? Then add 1.

- –1 = xFF (byte), xFFFF (word), xFFFFFFFF (doubleword)

- A negative 2’s complement hex’ integer begins with 8..F. Why?

- The range of unsigned integer values that can be stored in n bits is  0..2n-1. Byte : 0..255. Word : 0..65535.

- The range of signed integer values that can be stored in n bits is –2n-1..2n-1-1. Byte : -128..127. Word : -32768..32767.

Overflow

When performing integer arithmetic, an overflow condition is said to occur if the arithmetic produces a result that is outside of the range of the intended storage (see notes 3 and 4 above).

For example, suppose that we are performing byte arithmetic. The sum 125 + 125 will produce signed overflow but not unsigned overflow. The sum, 250, is within the unsigned byte range, but outside the signed byte range. It is important to understand that producing a carry indicates unsigned overflow, but not necessarily signed overflow. Consider the following examples carefully…

Binary		 Hex		Unsigned	Signed

(1)	 1010 1000	 xA8		168		-88

0010 1101	 x2D		 45	 	 45

===========	====		===	 	===

1101 0101	 xD5		213	 	-43	C = 0	V = 0

(2)	 1101 0011	 xD3		211	 	-45

1111 0100	 xF4		244	 	-12

===========	====		===	 	===

11100 0111	x1C7		455	 	-57	C = 1	V = 0

(3)	 0010 1101	 x2D		 45	 	 45

0101 1000	 x58		 88	 	 88

===========	====		===	 	===

1000 0101	 x85		133	 	133	C = 0	V = 1

(4)	 1101 0011	 xD3		211	 	-45

1010 1000	 xA8		168	 	-88

===========	====		===	 	===

10111 1011	x17B		379      -133	C = 1	V = 1

Note:

- Producing a carry, C = 1, indicates unsigned overflow.

- Producing a carry, C = 1, does not indicate signed overflow.

- To recognize signed overflow, two conditions must be present:

- the augend and addend must have the same sign, and

- the sum must have the opposite sign.